PatterOS

AI RIG — BILL OF MATERIALS & BUILD GUIDE

Three proven configurations for a local AI workstation, with a complete parts list and a step-by-step assembly walk-through.

WHO THIS IS FOR This guide builds the reference machine that the PatterOS install.sh script (coming soon!) is designed around: a 24 GB-class GPU on a low-cost, high-memory Xeon platform. Pick one of the three GPU options or another of your choice; all other components are shared in the build. No prior PC-building experience is assumed. If you can use a screwdriver and follow ten steps in order, you can build this.

THE GUIDING IDEA Memory is what matters most. A model either fits in memory or it doesn’t. Everything else — CPU speed, GPU model, disk — tunes how fast it runs, not whether it runs. That’s why this build pairs an inexpensive CPU and motherboard with as much RAM and VRAM as the budget allows. As I will often state throughout this project: 1 token per second is infinitely more than 0, and everything after that is just “profit”.

The Shared Platform

All three configurations use the same base. It’s deliberately built on a low-cost LGA2011-3 Xeon platform, which gives you a 12-core CPU, quad-channel memory and plenty of PCIe lanes for very little money.

This allows for more of the budget to be spent on the GPU and RAM, which is where AI performance actually comes from. This CPU and board are more than sufficient for a single GPU workstation, and it can support a 2-GPU setup with some creativity.

| Component | Choice | Why this part |
| --- | --- | --- |
| Motherboard + CPU | Machinist X99 PR9-H + Xeon E5-2650 v4 | 12 cores / 24 threads, quad-channel DDR4, multiple PCIe slots. Sold as a tested kit for under £50. |
| Memory (RAM) | 64 GB (4 × 16 GB) DDR4 | The single most important spec. 64 GB comfortably holds large models in system memory for CPU offload and leaves headroom for training data. |
| Storage | 512 GB NVMe SSD (PCIe Gen 3) | Fast model loading. 512 GB holds the OS plus several quantised models; add a second drive later for the full catalogue. |
| CPU cooler | TR-AXP120 low-profile + Arctic MX-7 paste | Keeps the Xeon cool and quiet; low-profile fits open benches and most cases. |
| Power supply | Corsair RM850e (850 W) | Headroom for a 24 GB GPU plus the platform, with quiet operation and good efficiency. |
| Chassis | Open-frame test bench | Easy to build on, excellent airflow, and trivial to swap GPUs. Move to a closed case later if you prefer. |
| Extras | CR2032 CMOS battery; optional AX210 Wi-Fi 6E card | Fresh CMOS battery avoids BIOS quirks on used boards; Wi-Fi card is optional if you don’t have Ethernet. |

MEMORY FIRST If you can stretch the budget anywhere, spend it on RAM and a bigger-VRAM GPU before anything else. A faster CPU barely moves AI performance; more memory directly decides which models you can run.

Choose Your GPU: Three Configurations

The only part that changes between the three builds is the graphics card. Pick the row that matches your budget and which software ecosystem you prefer. All three give you a 24 GB-class or capable card that PatterOS will detect and configure automatically.

| Configuration | GPU | VRAM | Approx. total | Best for |
| --- | --- | --- | --- | --- |
| 1 · NVIDIA (Flagship) | RTX 3090 Ti (Zotac) | 24 GB | £1,187 | Best all-round: CUDA, widest software support, and full training in Unsloth Studio. |
| 2 · AMD (Value) | RX 7900 XTX (Sapphire) | 24 GB | £1,009 | Most VRAM per pound. Inference via ROCm, training via Unsloth Core. |
| 3 · NVIDIA (Budget) | RTX 2070 Super (Asus) | 8 GB | £484 | Lowest cost entry. Runs the smaller models well; full CUDA support and Studio training. |

All three prices include the full shared platform above, with costs being indicative used-market prices in GBP that could vary.

1 · NVIDIA RTX 3090 Ti — the recommended build

If you can afford it, this is the one to get. 24 GB of VRAM runs the large models on the GPU, CUDA is the most mature AI stack. Inference and full fine-tuning in Unsloth Studio is fully supported. The 3090 Ti is a power-hungry card, which is why the build specifies an 850 W PSU at least.

2 · AMD RX 7900 XTX — the value 24 GB build

The same 24 GB of VRAM for noticeably less money. Inference through llama.cpp (built against ROCm) is excellent. For training, Unsloth Studio currently handles chat and data preparation on AMD, with full fine-tuning available through Unsloth’s code-based Core path. If your focus is running models rather than training them, this is superb value. AMD support is set to be added to Unsloth, if it hasn’t already by the time you are reading this.

3 · NVIDIA RTX 2070 Super — the budget entry

Proof that you don’t need to spend four figures. With 8 GB of VRAM it runs the smaller curated models comfortably on the GPU, and larger models still run using system RAM. Full CUDA support means Studio training works too. This is especially true now with the Gemma 4 12B multi-model release from Google.

A great way to start, with an easy upgrade path later, just swap the card when budget or necessity allow for it!

Full Bill of Materials

Every part, with an indicative price and a place to find it. The three GPUs are listed together; you only buy one. Everything else is common to all builds.

Shared platform (buy once)

| Item | Qty | Approx. £ | Find it |
| --- | --- | --- | --- |
| Machinist X99 PR9-H board + Xeon E5-2650 v4 kit | 1 | 47.02 | eBay search |
| SK Hynix 64 GB (4 × 16 GB) DDR4 | 1 set | 112.46 | eBay search |
| Micron 2210 512 GB NVMe (PCIe Gen 3) | 1 | 36.65 | eBay search |
| Corsair RM850e PSU (850 W) | 1 | 86.99 | Amazon |
| TR-AXP120 CPU cooler | 1 | 22.99 | Amazon |
| Arctic MX-7 thermal paste | 1 | 7.39 | Amazon |
| Open-frame test bench | 1 | 26.99 | Amazon |
| CR2032 CMOS battery | 1 | 3.33 | Amazon |
| AX210 Wi-Fi 6E + BT card (optional) | 1 | 22.49 | Amazon |

Shared platform subtotal: ~£344  (£366 with the optional Wi-Fi card).

Pick one GPU

| GPU option | Qty | Approx. £ | Find it |
| --- | --- | --- | --- |
| RTX 3090 Ti — Zotac Amp Extreme Holo 24 GB | 1 | 843.10 | eBay search |
| RX 7900 XTX — Sapphire Nitro+ Vapor X 24 GB | 1 | 665.38 | eBay search |
| RTX 2070 Super — Asus Dual Evo OC 8 GB | 1 | 140.38 | eBay search |

| Configuration total (platform + GPU) | Approx. £ |
| --- | --- |
| NVIDIA RTX 3090 Ti build | £1,187 |
| AMD RX 7900 XTX build | £1,009 |
| NVIDIA RTX 2070 Super build | £484 |

Building It: Step by Step

Work on a non-carpeted surface, and touch a bare metal part of the bench now and then to discharge static. Take your time, every step here is reversible so there is no rush. The order below is chosen so each part goes in while there’s still room to reach it, but feel free to change this if you are more comfortable with a different approach.

If you are following the video guide, you can watch ahead of your own progress for a sneak peak of what’s to come, so you can plan accordingly. This generally is a straightforward process, so don’t overthink it too much!

BEFORE YOU START Have the motherboard manual to hand, keep the tiny screws in a bowl, and do not plug the PSU into the wall until the very last step. The CPU cooler will prevent installation of all the RAM, so install RAM first. If using a larger GPU, then the WiFi card will not fit on this particular motherboard.

| 1 | Test bench / case Assemble the open-frame bench per its leaflet. Fit the motherboard stand-offs so they line up with the holes on your board. The bench keeps everything accessible and airflow generous while you build and test. |
| --- | --- |

| 2 | Motherboard Lay the motherboard on the stand-offs but don’t screw it down hard yet, leave it loose so you can press firmly when seating the CPU and cooler. Check the I/O side faces outward. |
| --- | --- |

| 3 | CPU Lift the CPU socket lever(s). Match the gold triangle on the Xeon to the triangle on the socket corner, lower it straight in with zero force, and close the lever. The kit usually ships with the CPU pre-installed and if so, just confirm it’s seated and the lever is latched. |
| --- | --- |

| 4 | RAM Open the clips on all four DIMM slots. Match the notch in each stick to the slot, then press each end down until the clips snap shut. Use all four sticks for quad-channel memory, the notch only lines up one way so take it easy when initially inserting the RAM stick and you can’t go wrong. |
| --- | --- |

| 5 | NVMe / SSD Find the M.2 slot, remove its little stand-off screw, slide the NVMe drive in at a slight angle, press it flat, and secure it with that screw. No cables needed for the NVMe. |
| --- | --- |

| 6 | CPU fan / cooler Put a pea-sized dot or X shape of Arctic MX-7 paste on the surface of the CPU. Lower the TR-AXP120 straight down onto the mounting points and tighten the screws a little at a time in a cross pattern so pressure stays even. Plug the fan into the CPU_FAN header. |
| --- | --- |

| 7 | PSU (Power Supply Unit) + initial motherboard cables Mount the PSU on the bench. Connect the big 24-pin motherboard cable and the 8-pin CPU power cable (top-left of the board). Leave the PSU switched off and unplugged. Don’t connect the GPU power leads yet. |
| --- | --- |

| 8 | GPU Release the clip on the top PCIe ×16 slot, line the card up and press until it clicks. Screw its bracket to the bench. Now connect the PCIe power cable(s) from the PSU to the card, ensuring you use the cables provided with your PSU. |
| --- | --- |

| 9 | Final check + button / aux wiring Fit the CR2032 battery and connect the front-panel power-switch header. Double-check: 24-pin in, 8-pin CPU in, GPU power in, RAM clips closed, cooler firm. Plug in a monitor (to the GPU), keyboard, and Ethernet. |
| --- | --- |

| 10 | Linux Mint OS install Switch the PSU on and press power. Enter the BIOS (usually Del or F2), confirm it sees all 64 GB of RAM and the NVMe, then boot from a Linux Mint USB stick and install to the NVMe. On first boot, run the PatterOS install.sh (coming soon) which will handle the entire installation, upgrade and first time service set up to turn your newly built rig into an AI workstation deployed on your local network |
| --- | --- |

Done — you’ve built a local AI workstation. Run install.sh (coming soon!) and you’ll be running your own models in minutes.

After the Build: First Boot

Once Linux Mint is installed, the PatterOS provisioning script takes over the software side completely.

When ready, this script and all other project files will be available in our project Github repository:

https://github.com/Daniel-Parke/YouTube-Repo

| Step | What to do |
| --- | --- |
| 1. Get install.sh (coming soon!) | Copy it onto the machine from your PatterOS source (USB stick or release page). |
| 2. Read it (MANDATORY) | Run “less install.sh”, it’s heavily commented and safe to read first. You should ALWAYS read a script before running it. NEVER trust unverified sources of code that you have not tested personally. |
| 3. Run it | “sudo bash install.sh”, then choose how much to install. It detects your GPU and other hardware automatically to determine the best set up. |
| 4. Reboot if asked | GPU drivers sometimes need one reboot. Re-running the script is always safe, and it will pick up from where it left off in the event of a crash. |
| 5. Start using it | Open the chat endpoint at http://YOUR-PC-IP:8080, and Unsloth Studio at port 7860. |

HARDWARE AND SOFTWARE, MATCHED This BOM and the PatterOS installer are designed together. The 24 GB cards line up with the larger curated models; the budget 8 GB card lines up with the smaller ones. Whichever you built, the installer sizes everything to your actual hardware automatically, and sets sensible defaults for each model when deployed using Llama.cpp.

UPGRADING LATER Because the platform is shared, upgrading is easy: drop in a bigger GPU, add a second one (the installer supports multi-GPU and mixed-vendor rigs), or even mix manufacturers. Although this may not be the optimal configuration, PatterOS will support it.